MERIDIAN INSTRUMENTS PVT. LTD.

AI Production Scheduler — Design & Data Reference

Transmitter Manufacturing · Order-to-Delivery · Three-Shift Operation

Companion to: Meridian_Instruments_Scheduler_Dataset.xlsx

Dummy company · sample data for demo & build · 21 July 2026

# 1. Company Snapshot

Meridian Instruments Pvt. Ltd. is a mid-size instrumentation manufacturer used as the dummy plant for this scheduler build. All figures below are realistic but invented; swap in a client's real numbers at onboarding (yellow cells in the workbook mark the editable inputs).

- Four product lines: Temperature (TT), Pressure (PT), Differential Pressure (DPT), Level (LT).

- Three shifts, 24-hour continuous operation, Monday to Saturday (26 working days/month).

- 15-stage order-to-ship routing. Lines share the admin/QC spine but differ in the middle: TT & LT are dry builds; PT & DPT are wet (oil fill + leak/pressure tests). Burn-in, calibration and helium-leak are the bottlenecks.

- Order-volume mix: PRESSURE = highest per month · LEVEL = lowest · DP and Temperature = average.

- ~99 direct production heads across three shifts; order entry, packing and dispatch are day-weighted.

# 2. Shift Model & Working Time

| Shift | Timing | Working days | Productive hrs |
| --- | --- | --- | --- |
| Shift-A | 06:00 – 14:00 | Mon–Sat | 7.4 h |
| Shift-B | 14:00 – 22:00 | Mon–Sat | 7.4 h |
| Shift-C | 22:00 – 06:00 | Mon–Sat | 7.4 h |
| Total / day | 24 h continuous | Mon–Sat | 22.2 h |

Conversion used by the engine: 480 min per shift − 30 min break, × 92% availability = 7.4 productive hrs/shift → 22.2 hrs/day → 26 days/month. Burn-in is wall-clock: it runs through nights and weekends regardless of shift.

# 3. Routing — Shared Spine + Line-Specific Stages

All four lines share an administrative and quality spine (marked ALL below). They diverge in the manufacturing middle: the sensor build differs by line, and the wet-line tests (helium leak, hydro) run only for pressure-bearing builds. Each stage carries setup time, per-unit time, and — the key addition for scheduling — operators and their active minutes per unit. The scheduler runs only the rows whose 'Applies to' matches the order's line.

| Stage | Name | Per-unit min | Operators | Active min | Applies to |
| --- | --- | --- | --- | --- | --- |
| 01 | Order Entry & Contract Review | 0 | 1 | 30 | ALL |
| 02 | Order Engineering / Config | 0 | 1 | 90 | ALL |
| 03 | Production Planning (MRP/ERP) | 0 | 1 | 45 | ALL |
| 04 | Procurement & Inbound Insp. | 0 | 1 | 45 | ALL |
| 05 | Kitting & Staging | 3 | 1 | 3 | ALL |
| 06a | Sensor: RTD/TC element build | 35 | 1 | 35 | TT |
| 06b | Sensor: single-chamber + diaphragm + fill | 50 | 2 | 50 | PT |
| 06c | Sensor: twin-chamber + dual diaphragm + fill | 70 | 2 | 70 | DPT |
| 06d | Sensor: radar probe / antenna | 55 | 2 | 55 | LT |
| 06H | └ Helium Leak Test | 12 | 1 | 12 | PT, DPT |
| 07 | Electronics Assembly | 35 | 1 | 35 | ALL |
| 08 | Final Assembly & Serialization | 22 | 2 | 22 | ALL |
| 09 | Calibration | 40 | 1 | 40 | ALL |
| 09B | └ Burn-in (wall-clock) | 2880 | 0.2 | 15 | ALL |
| 09T | └ Hydro / Proof-Pressure Test | 18 | 1 | 18 | PT, DPT |
| 09R | Rework / NCR (optional) | 90 | 1 | 90 | ALL |
| 10 | Special Certification | 25 | 1 | 25 | ALL |
| 11 | Final QC Inspection | 15 | 1 | 15 | ALL |
| 12 | Documentation Package | 8 | 1 | 8 | ALL |
| 13 | Packing | 10 | 1 | 10 | ALL |
| 14 | Shipping & Dispatch | 0 | 1 | 30 | ALL |
| 15 | Order Closure | 0 | 1 | 20 | ALL |

How the lines split: TT and LT are DRY builds — they skip helium leak (06H) and hydro (09T) entirely. PT is a single-chamber WET build; DPT is the twin-chamber WET build with the most demanding helium leak and longest calibration. Sensor build (06a–06d) is mutually exclusive: each order runs exactly one. Sensor/electronics (06/07) run in parallel; final assembly waits for both.

# 4. Per-Line Routing — Dry vs Wet

Because the builds differ, the lines don't run the same stage list. The table shows which stages each line actually runs and how many manufacturing stages that adds up to (excluding the shared spine).

| Line | Build type | Sensor sub-step | Helium leak | Hydro test | Mfg stages run |
| --- | --- | --- | --- | --- | --- |
| Temperature (TT) | Dry | RTD/TC element | No | No | 10 |
| Pressure (PT) | Wet | Single chamber + fill | Yes | Yes | 12 |
| Diff. Pressure (DPT) | Wet | Twin chamber + dual fill | Yes (critical) | Yes | 12 |
| Level (LT) | Dry | Radar probe / antenna | No | No | 10 |

## Per-unit time at shared stations (minutes)

Where a stage is common, the time still varies by line as the build gets heavier:

| Station | TT | PT | DPT | LT |
| --- | --- | --- | --- | --- |
| Kitting | 2 | 3 | 3 | 4 |
| Sensor build | 35 | 50 | 70 | 55 |
| Electronics | 28 | 32 | 38 | 38 |
| Final Assembly | 16 | 20 | 28 | 34 |
| Calibration (bottleneck) | 28 | 36 | 48 | 55 |
| Burn-in (hrs) | 24 | 48 | 48 | 60 |
| Final QC | 10 | 12 | 18 | 20 |
| Packing | 5 | 6 | 8 | 12 |
| Total active labour (min/unit) | 148 | 215 | 290 | 260 |

## Operators required at heavier stations

- Sensor build: TT = 1 operator · PT / DPT / LT = 2 operators.

- Final Assembly: TT / PT = 1 operator · DPT / LT = 2 operators.

- All other manned stations: 1 operator per unit. Burn-in: 0.2 (load/unload only).

# 5. Capacity — Three-Shift

Capacity is gated by the calibration bench (the manned pinch point), 3 benches per line. Theoretical figures assume every line runs flat-out at once; the realistic column applies an ~18% de-rate for shared burn-in chambers and final certification. Use the realistic figure when committing delivery dates.

| Line | Cal min/unit | Units/day (24h) | Units/mo (theoretical) | Realistic units/mo |
| --- | --- | --- | --- | --- |
| Temperature (TT) | 28 | 130 | 3,380 | 2,772 |
| Pressure (PT) | 36 | 101 | 2,626 | 2,153 |
| Diff. Pressure (DPT) | 48 | 76 | 1,976 | 1,620 |
| Level (LT) | 55 | 66 | 1,716 | 1,407 |
| PLANT TOTAL | — | 373 | 9,698 | 7,952 |

# 6. Monthly Order Mix

The sample Orders tab holds 30 live orders sized for a three-shift month. The mix is deliberately weighted so the demo shows the requested pattern:

| Line | Orders (sample) | Units (sample) | Volume rank |
| --- | --- | --- | --- |
| Pressure (PT) | 11 | 364 | HIGHEST |
| Temperature (TT) | 7 | 125 | Average |
| Diff. Pressure (DPT) | 6 | 101 | Average |
| Level (LT) | 6 | 35 | LOWEST |

# 7. Efficiency Markers

Markers to measure efficiency from order entry through delivery. OEE is the internal master metric (drives cost); OTIF is the customer-facing master metric (drives revenue). The composite Throughput Efficiency rolls capacity, quality and delivery loss into one balance-sheet proxy.

| Area / Marker | Definition | Ceiling | Target | Floor |
| --- | --- | --- | --- | --- |
| Order entry accuracy | Error-free ÷ total orders | 99% | 98% | 96% |
| OEE (master) | Availability × Performance × Quality | 85% | 75–80% | 60–65% |
| └ Availability | Run time ÷ planned time | 90% | 85% | 75% |
| └ Performance | Output ÷ rated capacity | 95% | 90% | 80% |
| └ Quality (FPY) | Good first-pass ÷ started | 99% | 96–97% | 94% |
| Bottleneck utilization | Busy ÷ available (cal+burn-in) | 90% | 80–85% | 70% |
| Rework rate | Reworked ÷ produced | <1% | 3–5% | <6% |
| OTIF (master) | On-time AND in-full ÷ orders | 98% | 95% | 90% |
| Throughput efficiency | On-time good units ÷ rated cap. | 82% | 72–80% | 65% |

## Highest achievable vs minimum acceptable

Ceiling: 85% OEE / 99% FPY / 98% OTIF is the practical world-class limit. 100% is impossible — changeover, preventive maintenance, calibration drift checks and mandatory burn-in dwell can't be zeroed. For a burn-in-heavy instrumentation shop, a sustainable ceiling is closer to 80–82% OEE.

Floor: ~65% OEE / 94% FPY / 90% OTIF is the minimum before the balance sheet is hit. Below it, expediting, overtime, swelling WIP and penalty clauses erode margin. The 65–80% band is where a good scheduler earns its keep — set the floor alarm at 65% OEE and the target band at 72–80%.

# 8. Disruption Scenarios (Reschedule Demo)

Seven what-if inputs the planner can type in plain English; the agent parses each into a structured constraint and re-optimises the queue.

- Machine breakdown — burn-in chamber down: queue doubles, low-priority orders slip, VIP protected.

- Material stockout — diaphragm lot fails inbound: only affected DP orders shift, rest holds.

- Rush order insertion — new top-priority order: slots ahead, two low-priority orders give way.

- Labour / shift loss — half the cal team sick: calibration backs up a day, downstream ripples.

- Priority bump — customer escalates an order: solver pulls it forward, others reschedule.

- Quality hold — batch fails hi-pot: rework loop inserted, delivery-at-risk flagged.

- Capacity boost — approve overtime: backlog clears, OTIF recovers.

# 9. Workbook Tab Map

| Tab | Contents |
| --- | --- |
| README | Plant snapshot and tab guide. |
| Routing_Master | Shared spine + line-specific stages, with an 'Applies to' column per line. |
| Line_TT / PT / DPT / LT | Each line's own routing — dry vs wet builds, per-unit time and manpower. |
| Line_Comparison | Side-by-side cycle time, labour and monthly order mix across all four lines. |
| Resources | Work-centres, parallel quantity, bottleneck flags and failure-logging rules. |
| Capacity | Three-shift capacity per line and plant total, theoretical and realistic. |
| Efficiency_Markers | Every efficiency marker with ceiling / target / floor bands. |
| Calendar | Three-shift pattern and working-time conversions. |
| Orders | 30 sample orders sized for a three-shift month. |
| Disruptions | Seven reschedule scenarios. |

Data caveat: operator counts and active-minute figures are realistic engineering estimates, not measured on a real floor. Confirm them with a time-and-motion pass at each station before committing delivery dates off this model.